TRANSMIT 1.0

DURRIDGE Company

February 1993

This program sends a file to the serial port for the purpose

of transmitting data from a PC to another device, such as

the DURRIDGE RAD7 instrument.

The syntax for its use is as follows. From the DOS prompt:

>TRANSMIT [filename] [COM<x>]

If a COM port is not given, then it automatically uses COM1

by default.  If you wish to send the file RAD0241.EXT to

COM1, for example, then type

>TRANSMIT RAD0241.EXT COM1

or

>TRANSMIT RAD0241.EXT

where COM1 is assumed by default.

If you wish to send the same file to COM2 instead, type

>TRANSMIT RAD0241.EXT COM2

and the data will go to COM2 instead.

This program always uses 1200 baud, 8 data bit, no parity,

and 1 stop bit.