CAPTURE 1.2

DURRIDGE Company

April 1998

This program has a very limited mission: to capture serial

port output from the RAD7 and put it into a file on your IBM

PC/XT/AT or compatible.  The same function can be found in

any data terminal program, but CAPTURE is a bit easier to

enter and exit from DOS.  But if you prefer to use a data

terminal program, by all means do so!

CAPTURE 1.2 was written and compiled in MicroSoft QuickBASIC

4.5.  Source code is provided for your use (CAPTURE.BAS), as

well as linkable object code (CAPTURE.OBJ), and a DOS

executable (CAPTURE.EXE).

To transfer data from the RAD7 to your computer, use a

null-modem RS-232 serial cable.  The RAD7 serial port

connector duplicates the IBM AT 9-pin serial port connector.

NITON can supply a null-modem cable to connect to either

9-pin or 25-pin serial port connectors (your choice).

Here is an example of CAPTURE in action.  From the DOS

prompt type:

C:>CAPTURE MYDATA.DAT COM1

where "MYDATA.DAT" is the name of the file I have chosen to

put the RAD7 data into, and "COM1" is the name of the serial

port that the RAD7 cable has been connected to.  You may use

any DOS file name for the output file, which means you are

limited to a maximum of an 8 character file name plus a 3

character extension.  Choose your file name carefully, as

CAPTURE will obliterate any existing file with the same

name.  For the sake of clarity, we recommend that you always

use the ".DAT" extension on your RAD7 output data file

names.  The serial port may be any from COM1 through COM4,

depending on how your computer is equipped.  If you leave

off the serial port name, CAPTURE will automatically use

COM1.  If you leave off the file name, CAPTURE will use

NITON.DAT as the output file.  CAPTURE responds to the above

command as:

*** CAPTURE 1.2 *** 04/98  Stephen Shefsky DURRIDGE Company.

Communication port COM1 is open.

Output file MYDATA.DAT is open.

Begin data transfer now.  Press (Q) key to quit or abort.

You have 30 seconds to initiate the data transfer from the

RAD7.  If the data transfer does not begin within 30

seconds, CAPTURE will "time out" and return to the DOS

prompt.  You may try CAPTURE again, of course.

To begin the output from the RAD7, go to the RAD7 key pad

and select ">Data Com " followed by the run number of the

data you wish to transfer.  You should immediately see lines

of numbers scrolling down the computer display.  When the

RAD7 finishes putting out data, the computer will beep and

display "Transfer finished."  If, at any time, you wish to

abort the data transfer, you may either press the RAD7's

menu key or your computer's "Q" key.

Now that you have the data in a file, you may do with it

whatever you wish.  DOS will allow you to do simple things

like copying or printing the data, but to do any sort of

sophisticated manipulation you will probably use a

spreadsheet program.